Mid-Term Evaluation Report

On

Reinforcement Learning-Based Dynamic Battery Thermal Management for Software Defined Electric Vehicles

Submitted in partial fulfillment of the

requirement for the award of the degree of

BACHELOR OF TECHNOLOGY

Session 2026-27

in

Computer Science and Engineering

By

Rishuraj Singh (2410030738)

Nihaal Sansanwal (2410031372)

Harshit Parmar (2410031422)

Chaitanya Sharma (2410031341)

Under the guidance of Dr. Pranav Srivastava

SCHOOL OF COMPUTER SCIENCE AND ENGINEERING

IILM UNIVERSITY, GREATER NOIDA

September, 2026

# CANDIDATE'S DECLARATION

This Mid-Term Evaluation report is submitted for the project entitled "Reinforcement Learning-Based Dynamic Battery Thermal Management for Software Defined Electric Vehicles", in partial fulfillment of the requirements for the degree of Bachelor of Technology, School of Computing Science and Engineering, IILM University, Greater Noida, under the supervision of Dr. Pranav Srivastava.

It presents a design review validating the system design against the working prototype, a summary of prototype/implementation progress, the experimental results obtained to date, and the material prepared for the Mid-Term Presentation and Viva, as required by the Major Project Evaluation Rubrics (around 14–19 September 2026).

Rishuraj Singh (2410030738)

Nihaal Sansanwal (2410031372)

Harshit Parmar (2410031422)

Chaitanya Sharma (2410031341)

This is to certify that the above statement made by the candidates is correct to the best of my knowledge.

Signature of Guide

Name: Dr. Pranav Srivastava

School: School of Computing Science and Engineering

IILM University, Greater Noida

# Table of Contents

| SL. No | Section | Page no. |
| --- | --- | --- |
|  | Introduction |  |
| 1 | Design Review |  |
| 2 | Prototype / Implementation Progress |  |
| 3 | Experimental Results |  |
| 4 | Mid-Term Presentation Outline |  |
| 5 | Anticipated Viva Questions & Answers |  |

# INTRODUCTION

This report supports the Mid-Term Evaluation of the RL-Based Dynamic Battery Thermal Management (RL-BTMS) project, whose assessment criteria (per the Major Project Evaluation Rubrics for B.Tech. Pre-final Year 2026-27) are: prototype or initial implementation, design validation, experimental results, technical presentation, and a question-and-answer session, together worth 20 marks. It draws directly on the Phase-II system design and the Phase-III implementation and results, reframing them for design validation and presentation purposes rather than repeating them in full.

| Metric | Verified result | Evidence file |
| --- | --- | --- |
| Hardness gate | Null cooling violates hot and fast-charge band | hardness_gate.json |
| PID tuning | 39 C, Kp 0.6, Ki 0, Kd 0 | pid_tuning.json |
| Shielded RL cost | PPO 1841.61; SAC 1768.50; TD3 1750.04 | policy_evaluation_summary.csv |
| Unshielded RL violation | PPO 42.28%; SAC 41.15%; TD3 42.08% | policy_evaluation_summary.csv |
| ONNX latency | 0.0170 ms median on laptop CPU | deployment_benchmark_sac_seed0.json |

Unshielded PPO, SAC and TD3 each violated the thermal band for more than 41% of evaluated time. This result demonstrates why a soft reward penalty cannot serve as a safety guarantee. The correct presentation is a supervised controller study with measured intervention frequency, rather than an independently safe RL result.

The current results replace the previous synthetic-cycle 95% energy-saving headline. On the eight fixed held-out episodes with the shield enabled, all controllers recorded zero band violation. PPO, SAC and TD3 required the shield for 22.70%, 19.63% and 21.85% of steps respectively. Their mean costs were 1841.61, 1768.50 and 1750.04, versus 1737.38 for the fairly tuned PID. The study therefore does not claim that RL has beaten PID at the present budget.

# 3. EXPERIMENTAL RESULTS

Measured pack validation remains open: conductances and pack hardware mass are engineering assumptions. The 20,000-step policies need longer and broader training before any superiority claim. The deployment result is a laptop CPU benchmark, not a vehicle ECU certification. The safety shield has not been formally verified beyond the simulated cases.

## 2.2 What remains

The working pipeline includes longitudinal road load, a first-order electrical model, four thermal nodes, independent cooling actuators, Gymnasium environment, fixed evaluation episodes, controller baselines, PPO/SAC/TD3 training and an ONNX/INT8 laptop deployment benchmark. The null-controller hardness gate demonstrates that doing nothing is unsafe in hot and fast-charge scenarios.

## 2.1 What is working

# 2. PROTOTYPE AND IMPLEMENTATION PROGRESS

| Design element | Rebuilt implementation | Review outcome |
| --- | --- | --- |
| Thermal state | Four nodes with coolant warming and spatial delta T | Corrects the one-node limitation |
| Action space | Continuous pump, fan and chiller commands | Implements multi-actuator coordination |
| Drive inputs | Real UDDS, HWFET, US06, ECE and EUDC traces with hashes | Synthetic inputs retired |
| Reward metric | True spatial delta T across nodes | Temporal rate proxy retired |
| Algorithms | PPO, SAC and TD3, five seeds each | Deep RL executed; DQN not included |
| Baselines | Tuned PID, bang-bang, null and sampling MPC | Fair PID tuning and MPC comparison added |
| Safety | Supervisory shield measured separately | Zero violation is conditional on intervention |

The rebuilt implementation deliberately corrects material deviations in the earlier prototype. The previous statement that no deviations existed is withdrawn. The original implementation used one thermal node, a scalar cooling command, synthetic profiles and a temporal max |dT/dt| proxy. Those choices could not implement the promised inter-cell gradient or multi-actuator coordination.

# 1. DESIGN REVIEW

# 4. MID-TERM PRESENTATION OUTLINE

Suggested slide-by-slide structure for the Mid-Term presentation (aim for 8–10 minutes + Q&A).

| Slide | Content | Source |
| --- | --- | --- |
| 1 | Title, team, guide | — |
| 2 | Problem: PID over-cools EV batteries, wastes energy; RL can adapt | Phase-I |
| 3 | System architecture (Fig. 1) and MDP design (Fig. 4) | Phase-II |
| 4 | What's implemented: battery model → env → PID → RL agent | Phase-III Sec. 1 |
| 5 | Training curve (Fig. M1) — show learning actually happened | Sec. 3 |
| 6 | RL vs. PID comparison (Fig. M2, Sec. 3.1 table) | Sec. 3 |
| 7 | Honest limitation: synthetic drive cycles, tabular RL as SB3 stand-in, US06 gradient trade-off | Sec. 2.2 |
| 8 | Next steps to Final Evaluation | Phase-III Sec. 5 |

# 5. ANTICIPATED VIVA QUESTIONS & ANSWERS

Q: Why is the reward function a weighted sum instead of a hard safety constraint?

A: A hard constraint would need constrained-RL methods (e.g. Lagrangian PPO); a weighted penalty is simpler to implement and tune first, and empirically it already produced zero safety violations across 160 evaluation episodes, so the added complexity wasn't justified for this proof-of-concept stage.

Q: Why does the RL agent have a higher temperature gradient than PID on US06?

A: The energy-cost term in the reward is currently large enough relative to the gradient term that the agent tolerates faster temperature swings to save cooling energy on the most current-intensive cycle. This is a reward-weighting issue, not a fundamental limitation, and is the top item for re-tuning before Final Evaluation.

Q: Why tabular Q-learning instead of DQN, as planned in Phase-II?

A: PyTorch/Stable-Baselines3 could not be installed in the offline development environment used to build the pipeline. Tabular Q-learning needed no external dependencies, so it let us build and validate the full environment-training-evaluation pipeline end-to-end now, while the SB3 script (DQN/DDPG/PPO/SAC) is written and ready to run on a GPU machine.

Q: How do you know the results aren't just noise?

A: The training curve shows a consistent, monotonic-on-average improvement over 400 episodes (mean reward −26.24 → −8.79), and the comparison uses 20 held-out episodes per cycle (not seen during training) with fixed seeds for reproducibility.

Q: Why synthetic drive cycles instead of the real NEDC/FTP-75/HWFET/US06 data?

A: The development sandbox had no internet access to download the official datasets. The synthetic profiles were designed to reproduce each cycle's qualitative character (stop-start, sustained cruise, aggressive bursts, mixed charging) so the pipeline could be built and tested now; replacing them with the real data is a drop-in change documented in drive_cycles.py.

Q: What happens if the battery temperature exceeds the critical limit during training?

A: The environment terminates the episode immediately once pack temperature exceeds 55°C (the CRITICAL_T constant), which both ends that episode's return early (a strongly negative outcome) and prevents the simulation from running in a physically meaningless regime.

Q: How is this different from just using a lower PID set-point?

A: PID with any fixed set-point applies the same control law regardless of driving context; the RL agent conditions its action on the full state (SoC, C-rate, ambient temperature, drive/charge mode), letting it apply different cooling strategies for, say, a hot-ambient highway cruise versus a cool-ambient urban stop-start cycle — which is exactly why its energy savings vary so much across cycles (97% on NEDC vs. 89% on US06).